This guide specification was prepared utilizing 3-part format recommended by the Construction Specifications Institute (CSI), and generally incorporates recommendations from their SectionFormat™/Page Format™, and MasterFormat™, latest Editions, insofar as practicable. Carefully review and edit the text to meet the Project requirements and coordinate this Section with the remainder of the Specifications and the Drawings. Where bracketed text is indicated, e.g. [text], make appropriate selection and delete the remainder of text within additional brackets, highlighting, and bold face type, if any. Consult the manufacturer for assistance in editing this guide specification for specific Project applications where necessary. This Specification was current at the time of publication but is subject to change. Please confirm the accuracy of these specifications with the manufacturer prior to use.

SECTION 07 72 33

ROOF ACCESSORIES

PART 1 - GENERAL

- SUMMARY

  - Section Includes:

    - Roof Hatches

      - RETROMOUNT™ SERIES ROOF HATCH

    - Accessories for Roof Hatches

      - SAFETY RAILINGS

      - SAFETY POSTS

  - Related Sections

- Section Includes: Furnish and install factory fabricated roof hatches.

- Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

  - Related Sections: Other specification sections which directly relate to the work of this 	section include, but are not limited to, the following:

- Section 05 50 00 - Metal fabrications for metal vertical ladders, ships’ ladders and stairs for access to roof hatches

- Section 07 01 40 – Maintenance roofing

- Section 07 01 70 – Operation and maintenance of roofing specialties and accessories

- Section 07 06 70 – Schedules for roof specialties and accessories

- Section 07 22 00 – Roof and deck installation

- Section 07 26 00 – Vapor retarders

- Section 07 27 00 – Air barriers

- Section 07 50 00 – Membrane roofing

- Section 07 81 00 – Roofing specialties

- Section 07 72 00 – Roofing accessories

- Section 07 72 43 – Roof walk boards

- Section 07 72 46 – Roof walkways

- Section 08 71 00 – Hardware: For lock type furnished by others

- Section 09 91 00 – Painting: For Field Painting of Roof Hatches

- Reference Standards:

    - American Society of Testing and Materials (ASTM) and Others:

      - ASTM A36/A36M-12, Standard Specification for Carbon Structural Steel

      - ASTM A153/A153M-09, Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware

      - ASTM A653/A653M-11, Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy Coated (Galvanized) by the Hot-Dip Process

      - ASTM A879/A879M-12, Standard Specification for Steel Sheet, Zinc Coated by the Electrolytic Process for Applications Requiring Designation of the Coating Mass on Each Surface

      - ASTM A1008/A1008M-12a, Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Solution Hardened, and Bake Hardenable

      - ASTM B209-10, Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

      - ASTM B221-12a, Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes

      - ASTM F2329-11, Standard Specification for Zinc Coating, Hot-Dip, Requirements for Application to Carbon and Alloy Steel Bolts, Screws, Washers, Nuts, and Special Threaded Fasteners

      - ASTM C1289-17, Standard Specification for Faced Rigid Cellular Polyisocyanurate Thermal Insulation

      - International Code Council (ICC)

    - International Building Code (IBC)

- ACTION SUBMITTALS

SPECIFIER: Revise Section number in the paragraph below to match that used in the Project Manual.

Submit in accordance with Section [01 33 00] [other]:

    - Product Data:

      - Materials description for roof hatches including details showing mounting type, relationships to surrounding construction, hatch type, materials, and construction, and locking features.

      - Installation instructions for each product specified.

    - Shop Drawings:

      - Include details of each hatch type, size and description of components, materials, attachment devices, description of frame and finish and construction details.

      - Schedule showing each type of hatch, locations, sizes, latching or locking provisions, and other data pertinent to installation

      - Indicate installation procedures and accessories required for a complete installation.

SPECIFIER: Delete subparagraph below for standard steel or galvanized steel hatches; retain for aluminum or other factory-finished products.

- QUALITY ASSURANCE

  - Regulatory Requirements:

    - OSHA 29 CFR 1910.23 Walking-Working Surfaces, Ladders

    - OSHA 29 CFR 1910.25 Walking-Working Surfaces, Stairways

    - OSHA 29 CFR 1910.28 Walking-Working Surfaces – Duty to have Fall Protection

    - OSHA 29 CFR 1910.29 Walking-Working Surfaces – Fall Protection Systems

    - International Building Code (IBC) Section 1015.7 Roof Access

    - International Building Code (IBC) Section 1011.12 Means of Egress, Stairway to Roof

    - International Building Code (IBC) Section 1011.13 Means of Egress, Guards

    - Comply with standards referenced in Article 1.03 REFERENCES.

    - Provide roof hatches produced by a single manufacturer.

- DELIVERY, STORAGE, AND HANDLING

  - Deliver, store, and handle roof hatches using means and methods that will prevent damage, deterioration, or loss.

  - Deliver units in manufacturer's original packaging, properly labeled for identification.

PART 2 - PRODUCTS

2.01	ROOF HATCH MANUFACTURERS

  - Acceptable Manufacturers:

Dur-Red Products – Part of Activar Construction Products Group, Inc.

9702 Newton Avenue S.

Bloomington, MN  55431

(800) 554-6077

www.activarcpg.com

info@activarcpg.com

  - Substitutions: Manufacturers seeking approval of their products are required to comply with the Owner's Instructions to Bidders, generally contained in the Project Manual.

2.02	RetroMount Series – Galvanneal Steel

- Style RHRMG: RetroMount, Galvanneal steel, insulated roof hatch cover and roof curb.

    - Model No. and Size:

      - RHRMG-1: 36 inches x 30 inches (914 mm x 762 mm)

      - RHRMG-2: 30 inches x 54 inches (762 mm x 1371.60)

      - RHRMG-4: 36 inches x 36 inches (914.40 mm x 914.40 mm)

      - RHRMG-5: 48 inches x 48 inches (1219.20 mm x 1219.20 mm)

      - RHRMG-WxL, where L is hinged side <Insert dimensions>

    - Performance Criteria: Tested to support a minimum live load of 40 pounds per square foot (195 kilograms per square meter).

    - Curb: 14 Gauge galvannealed steel

    - Cover: 14 Gauge galvannealed steel

    - Gasket: Continuous, weatherproof EPDM sponge rubber adhesive backed seal

    - Insulation: 1” polyisocyanurate in curb and cover with thermal resistance R-6

Hold-Open Arm: Clear zinc-plated automatic locking hold-open arm with red vinyl grip for one handed door operation

    - Latch: Clear zinc-plated steel slam latch with inside and outside handles and padlock hasps. Holds covers closed against a 20PSF wind uplift.

    - Finish: White textured powder coat paint

    - Hinge: Heavy duty zinc plated steel pintle hinge system

    - Springs: Powder coated, oil tempered, steel torsion spring provides counterbalanced door lift assistance

    - Hardware: Zinc-plated steel anchor tabs included with ¼-20x3/4” Philips head, self-tapping sheet metal screws for securing to inside of existing curb

2.03	RetroMount Series – Aluminum

  - Style RHRMA: RetroMount Aluminum, insulated roof hatch cover and roof curb.

    - Model No. and Size

      - RHRMA-1: 36 inches x 30 inches (914 mm x 762 mm)

      - RHRMA-2: 30 inches x 54 inches (762 mm x 1371.60)

      - RHRMA-4: 36 inches x 36 inches (914.40 mm x 914.40 mm)

      - RHRMA-5: 48 inches x 48 inches (1219.20 mm x 1219.20 mm)

      - RHRMA-WxL, where L is hinged side <Insert dimensions>

    - Performance Criteria: Tested to support a minimum live load of 40 pounds per square foot (195 kilogram per square meter).

    - Curb: 11 Gauge aluminum

    - Cover: 11 Gauge aluminum with 18 gauge aluminum cover liner

    - Gasket: Continuous, weatherproof EPDM sponge rubber adhesive backed seal

    - Insulation: 1” polyisocyanurate in curb and cover with thermal resistance R-6

    - Hold-Open Arm: Clear zinc-plated automatic locking hold-open arm with red vinyl grip for one handed door operation

    - Latch: Clear zinc-plated steel slam latch with inside and outside handles and padlock hasps. Holds covers closed against a 20PSF wind uplift.

    - Finish: Mill finish

    - Hinge: Heavy duty zinc plated steel pintle hinge system

    - Springs: Powder coated, oil tempered, steel torsion spring provides counterbalanced door lift assistance

    - Hardware: Zinc-plated steel anchor tabs included with ¼-20x3/4” Philips head, self-tapping sheet metal screws for securing to inside of existing curb

2.04	ACCESSORIES

  - Bolt-On Safety Railing

    - Style RSHWB: A bolt-on safety railing designed to bolt on the curb counterflashing and does not penetrate the curb protecting the integrity of the roof membrane.

      - Model No. and Size

        - RSHWB-1LY: 36 inches by 30 inches (914.4 mm by 762 mm)

        - RSHWB-2WY: 30 inches by 54 inches (762 mm by 1371.60 mm

        - RSHWB-4LY: 36 inches by 36 inches (914.40 mm by 914.40 mm)

        - RSHWB-5LY: 48 inches by 48 inches (1219.20 mm by 1219.20 mm)

        - RSHWB-WxL<Insert dimensions>

      - Performance:  Complies with OSHA CFR 29-1910.28 and CFR 29-1910.29.

        - 200LBS with max 3" deflection per OSHA 1910.29, ANSI 1264.4 , 50 PLF (pounds per linear foot) per 2024 IBC 1607.9 (ASCE 7)

      - Material: 1-⅝" x .065" (16 gauge) ASTM A513 galvanized steel tubing

      - Bracket Material: 2" x 2" x.048" (18 gauge) square galvanized steel tubing

      - Finish: Yellow powder coat

      - Hinges: Steel, adjustable spring hinges

      - Gate: Self-closing, self-latching with telescoping size adjustment

      - Latch: Steel, black powder coat pin hooks around gravity latch catch

      - Hardware: Black plastic tubing caps and zinc-plated steel hardware

- Ladder-Assist Safety Post (Extendable):  Preassembled steel safety post which attaches directly to the top two rungs (either square or round sized ¾” – 1-1/2”) of the vertical access ladder with two U-bolts and mounting bracket.

    - Model LP- 4

      - Post Assembly: Square post with latch which maintains the post at variable heights, and spring coil at base to allow user to lift (extend) post with one hand.

      - Post Finish: Safety yellow powder coat.

      - Weight: 25 lbs.  (12 kg).

D.	Ladder-Assist Safety Post – Extension-Up Series: Preassembled steel safety post which attaches directly to the top two rungs (either square or round sized ¾” – 1-1/2”) of the vertical access ladder with 2 U-bolts and mounting bracket.

            - Model RSLP-G

Post Assembly: Square post with release latch which maintains the post securely locked into the upright position, and spring coil at base to allow user to lift (extend) post with one hand.

Post Finish G: Hot-dipped galvanized.

PART 3 - EXECUTION

3.01	EXAMINATION

- Examine substrates for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

- Notify the Contractor in writing of conditions detrimental to proper and timely completion of the installation.

- Verify dimensions of roof openings for roof accessories.

- Proceed with installation only after unsatisfactory conditions have been corrected.

3.02	INSTALLATION

  - General: Comply with manufacturer's written instructions for installing roof hatches and accessories, if any.

    - Install roof hatches level, plumb, true to line and elevation, and without warping, jogs in alignment, buckling, or tool marks.

    - Anchor roof hatches securely in place so they are capable of resisting indicated loads.

    - Use fasteners, separators, sealants, and other miscellaneous items as required to complete installation of roof hatches and fit them to substrates.

    - Install roof hatches to resist exposure to weather without failing, rattling, leaking, or loosening of fasteners and seals.

  - Metal Protection: Protect metals against galvanic action by separating dissimilar metals from 	contact.

    - Coat concealed side of uncoated aluminum components with bituminous coating where in contact with wood, ferrous metal, or cementitious construction.

    - Underlayment:  Where installing roof hatches directly on cementitious or wood substrates, install a course of felt underlayment and cover with a slip sheet, or install a course of polyethylene sheet.

  - Installation: Install roof hatch so top surface of hatch curb is level.

SPECIFIER: Retain subparagraphs below if roof hatches are so equipped.

            - Attach safety railing system to roof hatch curb.

            - Attach ladder-assist post according to manufacturer's written instructions.

  - Surface Repairs:

SPECIFIER:  Select Galvanneal surfaces or factory-primed surfaces in subparagraphs below.

    - Galvanized Surfaces: Clean field welds, bolted connections, and abraded areas and repair galvanizing according to ASTM A 780.

    - Primed Surfaces:  Touch up factory-primed surfaces with compatible primer ready for field painting if scheduled.

3.03	ADJUSTING AND CLEANING

- Verify that roof hatches operate properly.

- Clean, lubricate, and adjust operating mechanism and hardware.

- Clean exposed surfaces according to manufacturer's written instructions.

- Replace roof hatches that have been damaged or that cannot be successfully repaired by finish touchup or similar minor repair procedures.

END OF SECTION